Traffic Light

Controller Using Sequential

Logic and Timers

Abstract

This project demonstrates the design and operation of an automatic traffic light controller using sequential logic and timer circuits. It provides an effective solution for controlling traffic at intersections with improved safety and efficiency.​

Introduction

Traffic lights play a crucial role in regulating vehicle and pedestrian movements at busy road intersections. Automated control is necessary to manage the increasing traffic in urban areas, prevent accidents, and reduce congestion.​

Objective

- To design a traffic light controller using digital logic and timers.

- To ensure proper sequencing of Red, Yellow, and Green signals.

- To provide reliable timing intervals between the signals.​

Components Required

- 555 Timer IC (Pulse generator)

- CD4017 Decade Counter IC (Sequencer)

- Red, Yellow, Green LEDs

- Resistors: 100kΩ, 22kΩ, 330Ω

- Capacitors: 1µF, 10µF, 2.2mF

- 1N4148 diodes

- 9V Battery

- Potentiometer

- Breadboard & connecting wires​

Methodology

The controller uses a 555 timer in astable mode to generate clock pulses. These are passed to the CD4017 counter, which activates its outputs sequentially, turning the LEDs ON and OFF in a predetermined order. Timing intervals are adjusted using resistors, capacitors, and a potentiometer.​

Circuit Description

- 555 timer: Generates clock pulses. The pulse frequency sets the timing of each signal.

- CD4017: Receives clock pulses and cycles through its outputs, each connecting to an LED.

- Diodes: Used with LEDs for proper signal flow control.

- Potentiometer: Allows for easy adjustment of light durations.​

Block Diagram Explanation

- 9V Battery supplies power.

- 555 Timer generates clock.

- Timer output is fed to the CD4017 counter.

- Counter outputs connect to Red, Yellow, Green LEDs through diodes and resistors.

- Optional: Pedestrian switch can add extra control (using another 555 timer).​

Working Principle

- 555 Timer produces a pulse that advances the counter.

- CD4017 counter stages activate corresponding LEDs (Red, Yellow, Green).

- Sequence and duration controlled via timing components.

- The cycle repeats for continuous operation.​

State Table

| State | Red | Yellow | Green | Time |
| --- | --- | --- | --- | --- |
| S0 | ON | OFF | OFF | 4 sec |
| S1 | OFF | ON | OFF | 1 sec |
| S2 | OFF | OFF | ON | 4 sec |
| S3 | OFF | ON | OFF | 1 sec |

Circuit Diagram Description

- Pin 3 of 555 timer connects to the clock input of CD4017.

- CD4017 outputs (Q0, Q1, Q2) drive the respective LEDs.

- Diodes are inserted between counter outputs and LEDs.

- Resistors are used to protect the LEDs.

- The circuit layout should follow standard breadboard assembly practices.​

Results and Testing

- The LEDs cycle correctly through Red, Yellow, and Green according to set intervals.

- The timer intervals can be verified using measurement tools.

- The system repeatedly demonstrates accurate sequential logic and timing.​

Conclusion

This sequential logic and timer-based traffic light controller provides robust, reliable control of signal cycles for road safety. It illustrates essential concepts in digital electronics and automation, offering simple expandability for more advanced applications.​

This structure includes all major headings required for a professional technical report on the topic. For practical circuit implementation, use the described connections and listed components