Introduction :

The Push Button Controlled LED Chaser is a simple yet conceptually rich experiment in the field of Digital Electronics and Logic Design (DELD). It illustrates the principle of sequential logic control using a microcontroller-based system, where the transition between states (LEDs) is triggered manually by user input through a push button.

The project utilizes an Arduino UNO as the main control unit. The Arduino reads the state of the push button, processes it in the program logic, and sends output signals to the connected LEDs. The program written in Arduino IDE defines how the LEDs respond to each button press, ensuring that only one LED remains ON at a time. Once the final LED in the sequence is reached, the system automatically loops back to the first LED, forming a continuous circular sequence.

At the hardware level, the circuit consists of five LEDs, one push button, resistors, and connecting jumper wires assembled on a solderless breadboard. The push button is connected to an input pin of the Arduino configured with an internal pull-up resistor to ensure stable input readings. Each LED is connected to a digital output pin through a 220Ω resistor to limit current and prevent damage to the LEDs.

This project was first simulated using TinkerCAD—a virtual electronics simulation platform—to verify the correctness of the circuit connections and logic. After successful simulation, the same design was implemented physically on a breadboard using the actual components. The transition from simulation to hardware testing provided valuable hands-on experience in circuit building, debugging, and microcontroller interfacing.

Literature Review :

1  Concept Overview:

- The project demonstrates sequential logic using a push button and Arduino UNO.

- Each button press acts as a manual clock pulse, advancing the LED sequence one step forward.

2  Role of Arduino UNO:

- Arduino UNO (ATmega328P) acts as the main controller for reading input and controlling outputs.

- The Arduino IDE is used to write a simple program that defines LED sequence behavior.

- Offers flexibility, easy reprogramming, and precise control compared to fixed hardware ICs.

3  Push Button Function:

- The push button provides manual user input to control LED transitions.

- Each press shifts the glowing LED to the next position.

- A debounce delay is added in code to avoid false triggering.

4  LED Operation and Resistors:

- LEDs are connected to Arduino output pins via 220Ω resistors for current limiting.

- Only one LED lights up at a time, representing the current state of the system.

- The sequence loops back to the first LED after the last one.

5  Comparison with Traditional Circuits:

- Unlike 555 Timer or CD4017-based chasers, this system uses manual control instead of automatic timing.

- Reduces hardware complexity and enhances learning through programming and real-time interaction.

6  Educational Relevance:

- Helps understand event-driven control, state transitions, and digital logic behavior.

- Provides hands-on experience in circuit assembly, debugging, and coding.

- Bridges digital electronics theory with practical microcontroller applications.

7  Summary:

- The project effectively combines hardware logic and software programming.

- Demonstrates sequential control, manual input processing, and LED state visualization.

- Ideal for learning the fundamentals of digital electronics and embedded systems.

Methodology / Working :

1  Circuit Design:

- Arduino UNO controls 5 LEDs connected to pins D2–D6 through 220Ω resistors.

- A push button is connected to pin D8 using the internal pull-up resistor.

2  Working Principle:

- Each button press turns OFF the current LED and turns ON the next one.

- After the last LED, the sequence resets to the first LED.

3  Software Implementation:

- Code written in Arduino IDE defines LED sequence and button input.

- A debounce delay is used to avoid multiple detections for a single press.

4  Simulation and Testing:

- Circuit first simulated on TinkerCAD, then implemented on breadboard.

- LEDs responded correctly to each button press in both setups.

5  Result:

- The project worked successfully, showing sequential LED control with manual input.

Working Principle :

- The circuit works on the sequential logic principle using Arduino UNO.

- Each button press sends a signal to Arduino to move to the next LED.

- The current LED turns OFF, and the next LED turns ON in sequence.

- After the last LED, the sequence resets to the first LED.

- A debounce delay in code ensures stable operation without false triggers.

- The process repeats continuously as long as the circuit is powered.

Hardware Required :

| Sr. No. | Component Name | Specification / Description | Quantity | Purpose / Function |
| --- | --- | --- | --- | --- |
| 1 | Arduino UNO | ATmega328P Microcontroller Board | 1 | Main control unit for LED and button logic |
| 2 | Breadboard | Solderless Type (Half/Full Size) | 1 | For connecting and testing all components |
| 3 | LEDs | 5mm (Any color) | 5 | Output indicators showing sequential operation |
| 4 | Resistors | 220Ω (Fixed) | 5 | To limit current to each LED |
| 5 | Push Button | 6mm Tactile Switch | 1 | Input for manual LED sequence control |
| 6 | Jumper Wires | Male-to-Male | As required | To make electrical connections on breadboard |
| 7 | USB Cable / Power Supply | 5V DC | 1 | To power the Arduino board |
| 8 | Laptop / Computer | With Arduino IDE installed | 1 | For programming and uploading code |
| 9 | TinkerCAD Software | Online Simulation Platform | 1 | For circuit design and testing before hardware |

Circuit Diagram/Code :

You can view and simulate the circuit here :

TinkerCAD Simulation link: https://www.tinkercad.com/things/eIZabFKvPRv/editel?returnTo=%2Fdashboard&sharecode=6DU0SQ3llKwWudcvk5cyOls1XPQrRS99dDmxxPPW7xA

Arduino Code :

const int clockPin = 2;

const int buttonPin = 3;

void setup() {

pinMode(clockPin, OUTPUT);

pinMode(buttonPin, INPUT_PULLUP);

digitalWrite(clockPin, LOW);

}

void loop() {

if (digitalRead(buttonPin) == LOW) {

digitalWrite(clockPin, HIGH);

delay(50);

digitalWrite(clockPin, LOW);

delay(200);

while (digitalRead(buttonPin) == LOW);

}

}

Result:

- The Push Button Controlled LED Chaser was successfully designed and implemented using Arduino UNO.

- Each button press correctly advanced the LED sequence while turning OFF the previous LED.

- The circuit worked smoothly in both TinkerCAD simulation and hardware testing.

- The debounce delay ensured accurate LED transitions without flickering.

- The system demonstrated proper sequential control and user interaction.

Conclusion:

- The project successfully shows how sequential logic can be controlled using manual input and microcontroller programming.

- It helped in understanding Arduino interfacing, digital I/O control, and event-driven operation.

- The use of Arduino simplified the design and reduced hardware complexity.

- The project bridged theoretical digital concepts with practical embedded applications.

- Overall, it provided valuable hands-on experience in simulation, coding, and hardware implementation.

Contribution:

| NAME | PRN | CONTRIBUTION |
| --- | --- | --- |
| Sanika Wasade | 24070521135 | Led hardware setup and breadboard wiring. Verified LED sequence and push button working in TinkerCAD and physical model. Managed testing and documentation. |
| Yash Khadgi | 24070521137 | Assisted in TinkerCAD simulation, verified circuit connections, and helped with formatting and report writing. |
| Parth Urkude | 24070521131 | Helped in Arduino coding, debugging sequence logic, and preparing literature review content. |
| Parth Kandikuwar | 24070521139 | Supported hardware testing, ensured LED timing accuracy, and assisted in troubleshooting and code refinement. |

Course Teacher:
Prof. Ankita Avthanka